Practical No:10

VLAN

on ASA

enable

configure terminal

interface vlan 1

nameif inside

ip address 192.168.1.1 255.255.255.0

security-level 100

no shutdown

exit

interface ethernet0/1

switchport access vlan 1

no shutdown

conf t

route outside 0.0.0.0 0.0.0.0 209.165.200.225

object network inside-net

subnet 192.168.1.0 255.255.255.0

nat (inside,outside) dynamic interface

end

conf t

class-map inspection_default

match default-inspection-traffic

exit

policy-map global_policy

class inspection_default

inspect icmp

service-policy global_policy global

exit

- Configuring DHCP, AAA, SSH

DHCP

dhcpd address 192.168.1.5-192.168.1.36 inside

dhcpd dns 209.165.201.2 interface inside

dhcpd enable inside

SSH

username admin password admin1

aaa authentication ssh console LOCAL

crypto key generate rsa module 1024

crypto key generate rsa modulus 1024

ssh 192.168.1.0 255.255.255.0 inside

ssh 172.16.3.3 255.255.255.255 outside

ssh timeout 10

- Creating DMZ VLAN

interface vlan 3

ip address 192.168.2.1 255.255.255.0

no forward interface vlan 1

nameif dmz

security-level 70

exit

interface ethernet 0/2

switchport access vlan 3

exit

show interface ip brief

- Creating object for dmz-server (nat)

conf t

object network dmz-server

host 192.168.2.3

nat (dmz,outside) static 209.165.200.227

exit

access-list OUTSIDE-DMZ permit icmp any host 192.168.2.3

access-list OUTSIDE-DMZ permit tcp any host 192.168.2.3 eq 80

access-group OUTSIDE-DMZ in interface outside

pcb to pcc allowed

pcb to server allwed

pcc to pcb not allowed

sever to pcb not allowed