Router>en

Router#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#enable

% Incomplete command.

Router(config)#configure terminal

^

% Invalid input detected at '^' marker.

Router(config)#

Router(config)#interface gigabitEthernet0/0

Router(config-if)#ip address 172.22.34.65 255.255.255.224

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#

Router(config)#interface gigabitEthernet0/1

Router(config-if)#ip address 172.22.34.97 255.255.255.240

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#

Router(config)#interface gigabitEthernet0/2

Router(config-if)#ip address 172.22.34.1 255.255.255.192

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#

Router(config)#end

Router#write memory

%LINK-5-CHANGED: Interface GigabitEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0, changed state to up

%LINK-5-CHANGED: Interface GigabitEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/1, changed state to up

%LINK-5-CHANGED: Interface GigabitEthernet0/2, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/2, changed state to up

%SYS-5-CONFIG_I: Configured from console by console

Building configuration...

[OK]

Router#

Router#conf t

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#access-list 101 permit tcp host 172.22.34.66 host 172.22.34.62 eq ftp

Router(config)#access-list 101 permit icmp host 172.22.34.66 host 172.22.34.62

Router(config)#access-list 101 permit tcp host 172.22.34.98 host 172.22.34.62 eq www

Router(config)#access-list 101 permit icmp host 172.22.34.98 host 172.22.34.62

Router(config)#access-list 101 deny icmp host 172.22.34.66 host 172.22.34.98

Router(config)#access-list 101 deny icmp host 172.22.34.98 host 172.22.34.66

Router(config)#access-list 101 permit ip any any

Router(config)#interface g0/0

Router(config-if)#ip access-group 101 in

Router(config-if)#exit

Router(config)#interface g0/1

Router(config-if)#ip access-group 101 in

Router(config-if)#exit

Router(config)#