enable

configure terminal

spanning-tree vlan 1 root primary

exit

enable

configure terminal

spanning-tree vlan 1 root secondary

exit

enable

configure terminal

interface range fa0/1 , fa0/3

spanning-tree guard root

exit

enable

configure terminal

interface range fa0/3 - 5

spanning-tree portfast

exit

//bpduguard

configure terminal

interface range fa0/3 - 5

spanning-tree bpduguard enable

exit

//rules

enable

configure terminal

interface range fa0/3 - 5

switchport mode access

switchport port-security

switchport port-security maximum 1

switchport port-security violation shutdown

switchport port-security mac-address sticky

exit